List of recognized occupational hazards in spaceflight facilities

Subcontractor must identify and provide training/treatment for the following as applicable:

| Hazard | Brief description |
| --- | --- |
| Asphyxiating/Hazardous Atmosphere | Nitrogen purges, tanks, backfilled/actively purged transportation containers, etc. |
| Chemicals | Hazardous, toxic or reactive chemicals - Includes IPA, ethanol for cleaning |
| Confined Space | Large enough for a person to enter, restricted space and egress, could contain hazardous atmosphere |
| Electrical/High Voltage | > 50 volts open/exposed energized circuitry |
| Elevated Work Area | Potential for fall, > 4 feet unprotected by guards or rails. Use of ladders, scaffolds, and/or mobile elevated work platforms - Fall protection plan required for use of Active Fall Protection - Competent person may be required |
| Ergonomic/Repetitive Motion | Excessive force, high repetition, contact stress, vibration, non-normal body position |
| Explosives | Pyros, ordnance, propellants. Always a personnel hazard, ESO oversight/approval required. |
| Flammable Atmosphere | Use of flammable gases or liquids. Ex: IPA, methane, hydrogen, etc. |
| Ground Support Equipment | Mechanical and electrical GSE design - may include Elevated Work Area and Fall Protection Plan |
| High Noise Environment | High impulse noise (>140 dB) or sustained high exposure (>85 dBA TWA) |
| High Temperature | High contact temperature or working in a high temperature environment |
| High-Capacity Batteries | Non-UL-listed Li-ion or Li-Po batteries, or presents a thermal runaway hazard |
| Lasers | Class 3B or Class 4 |
| Lifting Device operations, suspended loads | Cranes, forklifts, manlifts, hoists, proof tests, personnel protection |
| Lifting devices, personnel in basket | Fall protection/restraint, proof tests |
| Low Temperature, cryogens | Cryogenic materials (LN2, LH2, LHe, LOX, solid CO2, and H2, etc.) or working in a cold environment |
| Mechanisms with Stored Energy | Springs, tensioned cables, pressurized glass shattering, etc. |
| Physical Hazards, Machines | Pinch-point hazards, rapidly-moving parts, rotating parts |
| Pressure/Vacuum System | Pressure systems, bottles, tanks, cylinders, GN2 and LN2, etc. |
| Radiation (Ionizing) | Radioactive material or x-ray sources |
| Radiation (RF, UV, IR) | Com antennas, radar, high-intensity UV or IR |
| Transportation | Hazardous materials, asphyxiating/hazardous atmospheres |
| Hazardous Waste | Includes hazardous (chemical) waste (and wipes) and Universal waste (batteries). |